Packet Tracer Office Network

Final Lab Report

Kyle Wooten School Project Portfolio

| Item | Details |
| --- | --- |
| Project Folder | 01-Networking / Packet-Tracer-Office-Network |
| Platform | Cisco Packet Tracer |
| Primary Devices | Cisco 2911 routers, Cisco 2960 switches, PCs, wireless access points |
| Network Design | Head Office and Branch Office enterprise-style network |
| Primary Skills | VLANs, trunking, router-on-a-stick, inter-VLAN routing, ACLs, DHCP/DNS planning, testing |
| Report Purpose | Document design, configuration, testing, troubleshooting, and final project outcome |

## Disclaimer

This report documents a Cisco Packet Tracer educational lab completed for training and portfolio development. All configurations and testing were performed in a simulated lab environment.

# 1. Executive Summary

This project involved designing and documenting a complete office network in Cisco Packet Tracer. The network connected a Head Office and Branch Office using Cisco routers and switches, department VLANs, router-on-a-stick inter-VLAN routing, trunk links, wireless access, guest segmentation, and access control planning.

The project demonstrates practical networking fundamentals required for IT support, network administration, cybersecurity lab work, and technical troubleshooting. It also includes lessons learned from configuration errors, especially the proper order of configuring 802.1Q encapsulation before assigning IP addresses to router subinterfaces.

# 2. Lab Environment

| Component | Description |
| --- | --- |
| Simulation Platform | Cisco Packet Tracer |
| Routers | Cisco 2911 |
| Switches | Cisco 2960 |
| End Devices | Department PCs, wireless clients, optional servers |
| Head Office Departments | Admin, HR, IT |
| Branch Office Departments | Sales, Support |
| Security Segment | Guest WiFi VLAN |
| Example Head Office Network | 192.168.100.0/24 |
| Example Branch Office Network | 192.168.200.0/24 |

# 3. Project Objectives

Design a complete office network with a Head Office and Branch Office.

Use Cisco 2911 routers and Cisco 2960 switches.

Create VLANs for multiple departments.

Assign switch ports to the correct VLANs.

Configure trunk links between switches and routers.

Configure router-on-a-stick subinterfaces using 802.1Q encapsulation.

Enable inter-VLAN routing for authorized internal communication.

Add wireless access and guest WiFi segmentation.

Use ACLs or access restrictions to prevent guest access to internal departments.

Test and document connectivity between devices and networks.

Troubleshoot configuration errors and document the resolution.

# 4. Network Topology Summary

The topology was designed as a two-site office network. The Head Office included Admin, HR, and IT departments. The Branch Office included Sales and Support departments. Each department was separated using VLANs, and each VLAN used a router subinterface as its default gateway.

Cisco 2960 switches provided access-layer connectivity for department PCs. Cisco 2911 routers provided routing between VLANs and between office sites. Wireless access was included for users, and a guest WiFi VLAN was planned to isolate untrusted guest devices from internal business systems.

# 5. VLAN and IP Addressing Plan

| VLAN ID | Name | Location | Purpose | Example Network |
| --- | --- | --- | --- | --- |
| 10 | Admin | Head Office | Administrative users | 192.168.100.0/27 |
| 20 | HR | Head Office | Human Resources users | 192.168.100.32/27 |
| 30 | IT | Head Office | IT support and administration | 192.168.100.64/27 |
| 99 | Management | Head Office | Network management | 192.168.100.96/27 |
| 100 | Guest WiFi | Head Office | Guest wireless access | 192.168.100.128/27 |
| 40 | Sales | Branch Office | Branch Sales users | 192.168.200.0/27 |
| 50 | Support | Branch Office | Branch Support users | 192.168.200.32/27 |
| 199 | Branch Mgmt | Branch Office | Branch network management | 192.168.200.64/27 |

# 6. Work Performed

## 6.1 Topology Build

Placed routers, switches, PCs, wireless access points, and optional server devices in Packet Tracer.

Organized the topology into Head Office and Branch Office sections.

Connected devices using appropriate copper straight-through or serial links depending on the device role.

## 6.2 VLAN Creation

Created department VLANs on Cisco 2960 switches.

Named VLANs according to department role.

Verified VLAN configuration with show vlan brief.

## 6.3 Access and Trunk Port Configuration

Assigned end-user PC switch ports to the correct department VLAN.

Configured trunk links between switches and router-facing interfaces.

Verified trunk status with show interfaces trunk.

## 6.4 Router-on-a-Stick Configuration

Created router subinterfaces for each VLAN.

Applied encapsulation dot1Q before assigning IP addresses.

Used subinterface IP addresses as the default gateways for VLAN clients.

## 6.5 Inter-VLAN and Branch Routing

Configured routing to allow authorized communication between department networks.

Validated routing tables and interface status.

Tested connectivity between Head Office and Branch Office clients.

## 6.6 Wireless and Guest Segmentation

Added wireless access where required.

Placed guest users into a separate guest VLAN.

Designed restrictions so guest traffic could not access internal department networks.

## 6.7 ACL and Security Testing

Planned or configured ACLs to restrict guest VLAN access to internal VLANs.

Tested guest-to-internal connectivity to confirm restrictions.

Verified authorized internal traffic still functioned.

## 6.8 Connectivity Validation

Performed PC-to-gateway, same-VLAN, inter-VLAN, and branch-to-head-office ping tests.

Used show commands to validate VLANs, trunks, routes, and ACLs.

Documented successful and blocked traffic flows.

# 7. Network Testing Results

| Test | Source | Destination | Expected Result |
| --- | --- | --- | --- |
| Gateway Ping | Admin PC | Admin VLAN Gateway | Success |
| Gateway Ping | HR PC | HR VLAN Gateway | Success |
| Gateway Ping | IT PC | IT VLAN Gateway | Success |
| Same VLAN Ping | Admin PC 1 | Admin PC 2 | Success |
| Inter-VLAN Ping | Admin PC | IT PC | Success if allowed |
| Branch Connectivity | Sales PC | Head Office Router | Success |
| Branch-to-HQ Ping | Support PC | Admin PC | Success if allowed |
| Guest Isolation | Guest WiFi PC | HR or Admin PC | Blocked |

# 8. Key Verification Commands

| Command | Purpose |
| --- | --- |
| show vlan brief | Verify VLAN creation and access port membership. |
| show interfaces trunk | Verify trunk links and allowed VLANs. |
| show ip interface brief | Verify router and switch interface status. |
| show running-config | Review current device configuration. |
| show ip route | Verify routing table and reachable networks. |
| show access-lists | Verify ACL entries and traffic restrictions. |
| ping | Test host, gateway, VLAN, and branch connectivity. |

# 9. Issues Encountered and Resolutions

| Issue | Cause | Resolution |
| --- | --- | --- |
| Router subinterface IP error | IP address was attempted before VLAN encapsulation was configured. | Configured encapsulation dot1Q first, then applied the gateway IP address. |
| VLAN connectivity failure | Access ports were missing or assigned to the wrong VLAN. | Reviewed show vlan brief and corrected switchport access VLAN assignments. |
| Trunk not carrying VLAN traffic | Trunk mode or allowed VLANs were not configured correctly. | Configured switchport mode trunk and verified with show interfaces trunk. |
| Branch-to-HQ failure | Routing or IP addressing was incomplete. | Reviewed interfaces, routes, and ping tests; corrected gateway or route configuration. |
| Guest reaching internal networks | Guest VLAN restriction was not applied or tested. | Applied ACL restrictions and validated blocked traffic to internal VLANs. |

# 10. Skills Demonstrated

Cisco Packet Tracer topology design.

Cisco 2911 router configuration.

Cisco 2960 switch configuration.

VLAN creation and verification.

Access and trunk port configuration.

802.1Q router-on-a-stick configuration.

Inter-VLAN routing.

IP addressing and subnet planning.

Branch office connectivity.

Wireless and guest network segmentation.

ACL-based traffic restriction planning.

Network testing and troubleshooting.

Professional technical documentation.

# 11. Final Outcome

The final Packet Tracer Office Network successfully represented a small enterprise network with separate department VLANs, routed connectivity, Head Office and Branch Office communication, wireless support, guest segmentation, and documented testing. The project produced a portfolio-ready demonstration of core networking skills and Cisco configuration concepts.

The completed deliverables include a README.md file, final lab report PDF, recommended commands file, topology file placeholder, screenshots, IP addressing table, and testing results documentation.

# 12. Reflection

This project reinforced that successful network design depends on careful planning before configuration. VLANs, gateways, trunk links, router subinterfaces, IP addressing, and ACLs all depend on each other. The most important troubleshooting lesson was learning the correct order for router subinterface configuration: 802.1Q encapsulation must be configured before an IP address can be applied.

The project also showed the value of testing after each stage. Verifying VLANs, trunks, routing, gateways, and ACLs helped isolate problems quickly and produced documentation that can be used for a resume, GitHub portfolio, or job interview discussion.